Supplement Information:

Transformation Procedures for Coordinates, Displacements, and Velocities Between Global and Regional Reference Frames

Guoquan Wang, Yan Bao

In geodesy, the transformation of GNSS position time series between reference frames is primarily achieved through two strategies: (i) a daily 7-parameter approach and (ii) a time-independent 14-parameter approach, both based on the Helmert transformation between ECEF- XYZ coordinates. The daily 7-parameter method calculates a new set of transformation parameters (three translations: Tx, Ty, Tz; three rotations: Rx, Ry, Rz; one scale factor s) for each daily solution. While rigorous, this method is often cumbersome for end-users in applied fields who require a straightforward, long-term stable reference frame without the overhead of daily parameter management. To address this, the geodetic community employs a time-independent 14-parameter transformation method, which comprises the seven standard Helmert parameters at a specific epoch and their constant rates of change over time, including Tx, Ty, Tz, Rx, Ry, Rz, s and their rates Tx', Ty', Tz', Rx', Ry', Rz', s'.

The 14-parameter method is based on the assumption of the linearity of the tectonic plate moment, in turn the linear changes of these 7 parameters over time. Over the past decade, the 14-parameter Helmert model has been further simplified to a “one-set 7-parameter” formulation that aligns two reference frames at a chosen epoch t0 (Wang et al., 2013). Specifically, we impose that at t0 all sites have identical XYZ coordinates in the global and regional frames, so these 7 instantaneous Helmert parameters at t0 are zero. t0 is set as 2020.0 for realizing regional reference frames in China. In practice, because both frames are realized from the same GNSS measurements and instrumentation, no inter-frame scale change is expected; therefore s'=0. The precise transformation of the XYZ coordinates is then described by the one-set 7-parameter: t0, Tx', Ty', Tz', Rx', Ry', and Rz'. The detailed method for calculating the 7-parameters for realizing a stable regional reference frame can be referred to Wang et al. (2014).

This one-set 7-parameter transformation greatly simplifies use: a fixed parameter set lets users transform multi-year position time series with one formula, eliminating daily re-estimation. It is well suited for long-term studies of subsidence, faulting, and other deformation processes.

1. Transforming ECEF-XYZ between global and regional reference frames

The one-set 7-parameter Helmert transformation defines the relationship between the regional and global frames by aligning their origins and orientations at a single reference epoch, t0, while enforcing a unit scale factor (i.e., zero scale difference) between the two systems. Let the position vector in the global frame at time t be:

Pt=X(t)Y(t)Z(t), T'=Tx'Ty'Tz', R'=Rx'Ry'Rz',

∆t=t-t0.

Where Tx', Ty', Tz' are translation rates in m/year; Rx', Ry', Rz' are rotation rates in rad/year. With the one-set 7-parameter method, the forward transformation from a global (abbreviated as G) to regional (abbreviated as R) refence frame is

PRt=PGt+∆tT'+∆tR'×∙PG(t)               (1)

where the skew-symmetric cross-product matrix R'x is

R'x=0Rz'-Ry'-Rz'0Rx'Ry'-Rx'0                                 (2)

Explicitly,

X(t)R=X(t)G+Tx'∙(t-t0)+Rz'∙(t-t0)∙Y(t)G-Ry'∙(t-t0)∙Z(t)G

Y(t)R=Y(t)G+Ty'∙(t-t0)-Rz'∙(t-t0)∙X(t)G+Rx'∙(t-t0)∙Z(t)G         (3)

Z(t)R=Z(t)G+Tz'∙t-t0+Ry'∙t-t0∙XtG-Rx'∙t-t0∙YtG

where XtG, YtG, and ZtG are the ECEF-XYZ coordinates (at epoch t) of a site with respect to IGS20; XtR, YtR, and ZtR are the XYZ coordinates of the site with respect to the regional reference frame. The units of these XYZ coordinates are meters. t0 is the epoch that aligns the coordinates with respect to these two reference frames. Tx', Ty', Tz', Rx', Ry', and Rz' are constant parameters indicating the rates (one-time derivates) of translational shifts and rotations along the x, y, an z coordinate axes from IGS20 to the regional reference frame.

The inverse transformation from the regional reference frame to IGS20 can be estimated using the following equations:

X(t)G ≈  X(t)R-Tx'∙t-t0-Rz'∙t-t0∙YtR+Ry'∙(t-t0)∙Z(t)R

YtG ≈ YtR-Ty'∙t-t0+Rz'∙t-t0∙XtR-Rx'∙t-t0∙ZtR           (4)

Z(t)G≈Z(t)R-Tz'∙t-t0-Ry'∙t-t0∙XtR+Rx'∙t-t0∙YtR

A Python program, Global-Regional_XYZ_Transform.py, is provided to perform the coordinate transformation between IGS20 and regional reference frames, following the methodology outlined above.

2. Transforming ENU displacements between global and regional reference frames

In practice, users typically work with local displacement time series in the East-North-Up (ENU) directions derived from IGS20 solutions. These can be transformed into a regional reference frame by applying the one-set 7-parameter transformation to their ECEF-XYZ coordinates as an intermediate step. The procedure is:

Step 1. Estimate ECEF-XYZ at the site

A geographical location of the site defined by longitude (λ) and latitude (ϕ) can be estimated from the site log or other sources. Small datum differences (e.g., WGS84 vs. IGS20) or ellipsoidal height errors of a few meters introduce negligible changes in the local ENU rotation and the resulting transformed displacements or velocities. For most practical applications, approximate station latitude and longitude (e.g., from Google Earth) are sufficient to transform ENU displacement time series or site velocities between global and regional reference frames.

From the geodetic longitude (λ), latitude (ϕ), and ellipsoidal height (h), the ECEF-XYZ coordinates P0=(X0, Y0, Z0) in the global reference frame can be estimated according to the following equations (Eq. 5). If the ellipsoidal height is negligible or unknown, it can be assumed zero (or an approximate value).

X0=(N(ϕ)+h) cosϕ cosλ

Y0=(N(ϕ)+h) cosϕ sinλ                     (5)

Z0=(N(ϕ)1-e2+h)sinϕ

where N is the prime-vertical radius of curvature:

N(ϕ)=a1-e2sin2ϕ , a=6378137.0 m (GRS80 semi-major axis), f=1/298.257222101 (flattening), and e2=2f-f2 .

Step 2. Convert ENU displacements to XYZ

Convert each epoch’s ENU displacements to XYZ displacements using the local rotation.

ΔrXYZG=RENU2XYZ×ENU           (6)

ΔrXYZG represent a 3-dimensional (3D) displacement vector in the ECEF-XYZ coordinate system, i.e., its components are (ΔX, ΔY, ΔZ). RENU2XYZ represents a 3×3 rotation matrix that transforms a local ENU vector into the XYZ coordinate system at the station’s reference point (X0, Y0, Z0):

RENU2XYZ=-sinλ-sinλcosλ​cosϕcosλcosλ-sinϕsinλcosϕsinλ0cosϕsinϕ.                           (7)

Then form absolute ECEF-XYZ coordinates at epoch t:

PGt=P0+ΔrXYZG(t)……   (8)

Step 3. Transform XYZ from global to regional reference frame

The ECEF-XYZ coordinates can be transformed to regional frame according to Eq. (3), applying the one-set 7-parameters.

Step 4. Convert XYZ to ENU within the regional reference frame

Subtract the original reference point (X0, Y0, Z0) to get the displacements in XYZ coordinates:

ΔrXYZR=(XR-X0, YR-Y0, ZR-Z0)                   (9)

Rotate these displacements back into the ENU frame:

ERNRUR=RENU2XYZT×ΔrXYZR                      (10)

This yields the local ENU displacement time series with respect to the regional reference frame. RENU2XYZT is the transpose of the matrix RENU2XYZ, commonly denoted as RXYZ2ENU.

The reverse transformation of ENU displacement time series from regional to global reference frame follows the same logic but applies the inverse XYZ transform (Eq. 4) in Step 3, followed by the same ENU/XYZ rotations.

A Python program, Global-Regional_ENU-Displacement_Transform.py, is provided to transform ENU displacement time series between the global and regional reference frames, implementing the methodology described above.

3. Transforming ENU velocities between global and regional reference frames

For practical applications, it is often necessary to convert site velocities between regional and global reference frames. This transformation of ENU velocities can be accomplished through the following steps:

Step 1: Estimate position at the alignment epoch
The initial ECEF-XYZ position at the site P0 defined by longitude (λ) and latitude (ϕ) can be estimated according to Eq. 5.

Step 2: Convert ENU-Velocities to XYZ-Velocities within the global reference Frame

This can be done by

VXYZG=RENU2XYZ×VENUG                          (11)

RENU2XYZ is defined in Eq. 7.

Step 3. Convert VXYZG to VXYZR

According to Eq. 1, the XYZ velocities with respect to the regional reference frame can be obtained by differentiating the one-set 7-parameter position transform:

VXYZR=VXYZG+T'-R'×P0.                                (12)

Step 4. Convert VXYZR to VENUR

Finally, convert the ECEF velocities in the regional frame to ENU velocities:

VENUR=RENU2XYZT×VXYZR                                  (13)

Reverse conversion from regional to global frame of ENU site velocities follows the same steps, but using the reverse XYZ-velocity transform in Step 3:

VXYZG=VXYZR-T'+R'×P0                                (14)

A Python program, Global-Regional_ENU-Velocity_Transform.py, is provided to transform site velocities between the global and regional reference frames in the ENU directions, implementing the methodology described above.

The Python programs are archived on the first author’s GitHub repository: https://github.com/bob-Github-2020/China_Regional_ReferenceFrames